| Item | Part no | Drg no | Year | Rim | Features | Parts required |
| --- | --- | --- | --- | --- | --- | --- |
| B/plate | 4080-26 | C14/5 | 26-27 | x | Has 29/64 hole for brake anchor pin (not listed), a grease retainer plate attached to inside of plate by 6 rivets, a ¼ dia hole positioned at the bottom of the grease plate, long brake cam with oil hole pointing towards centre of plate and a riveted in fulcrum pin. Although spindle diameter changed in 1927 to 9/16 this was accommodated by a modified brake plate lock nut rather than a different plate. | C13/9-26 C13/10-26 |
| B/plate | 4080-28 | C14/5 | 28 | x | Hole for brake anchor pin (4180-28) improved by addition of a bush, grease retainer plate and ¼ hole deleted. In spite of this plate not fitting earlier forks Ariel failed to give it a different number. | C13/9-26 C13/10-26 C14/26 |
| B/plate | 4080-29 | C14/56 | 29 | x | T/arm position altered and now on a stud | C13/9-26, C13/10-26, C14/58A |
| B/plate | 4080-30 | C14/56A | 30-31 | chrome | Dust rim added. | C13/9-26, C13/10-26, C14/58A, 4088-30 |
| B/plate | 4080-32 | C15/41C | 32 | y | T/arm in usual place but b/a at 3 o'clock, fulcrum adj |  |
| B/plate | 4080-33 | C14/56AC | 33-37 | black | T/arm position altered to move brake plate clockwise by 5.5 deg, no fulcrum adj | C13/9-33, C13/10-33, C14/58A, 4088-30 |
| B/plate | 4080-34 | C14/56ACK | 34-37 | chrome |  |  |
| B/plate | 4080-38 | C15/98C | 38-46 | black | Floating b/cam | C13/9-38, C13/10-38, 4088-30 |
| B/plate | 4085-38 | C15/98C | 38-40 | chrome |  |  |
| B/plate | 4081-38 | C15/98AC | 38-46 | black, | FWD speedo | C13/9-38, C13/10-38, 4088-30 |
| B/plate | 4081K-38 | C15/98ACK | 38-40 | chrome |  |  |